[image: image6.png]meridian BIOSCIENCE
S





Page 9 of 10

Procedure and revision number: 0095023 Rev. H  CLSI 07/15

Procedure:  ImmunoCard STAT Cryptosporidium/Giardia Method
Meridian Catalog 750830
	Institution: 

	Address:

	Department:


	Prepared By:
	Date Adopted
	Supersedes procedure #:

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	


	Distributed To:
	# of Copies
	Distributed To:
	# of Copies

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	


NOTE: This procedure is provided to Meridian’s customers to assist with the development of laboratory procedures.  This document was derived from, and was current with, the instructions for use (IFU) that accompanies the product at the time it was created. The user is instructed to consult the IFU packaged with the product to ensure currency of the procedure prior to adapting the document to routine laboratory use and periodically thereafter to ensure future IFU modifications, which might effect this procedure, are identified.   Any modifications to this document are the sole responsibility of the person making the modifications.

PRINCIPLE:

The ImmunoCard STAT! Crypto/Giardia test is a rapid immunoassay for the qualitative detection of Cryptosporidium parvum and Giardia lamblia specific antigens in aqueous extracts of human fecal specimens.  It is intended for in vitro diagnostic use as an aid in the detection of suspected Cryptosporidium and Giardia infections by professional laboratories.

The ImmunoCard STAT! Crypto/Giardia test is a qualitative immunochromatographic assay that simultaneously detects and distinguishes between Cryptosporidium and Giardia antigens in aqueous extracts of patient stool specimens. The specimen, collected in a sample transport or preservative medium, is added to a tube containing a treatment buffer. A biotinylated anti-Giardia capture antibody reagent is then added, followed by a pooled suspension of colloidal dye labelled monoclonal antibodies to Giardia and Cryptosporidium. The sample is then mixed and poured into the test device that contains a capture reagent (an avidin derivative) for Giardia, a capture antibody for Cryptosporidium, and a control antibody that binds to excess colloidal dye conjugate. If Giardia antigen is present in the sample, a grey-black band will develop at the GIAR position in the device window. If Cryptosporidium antigen is present, a grey-black band will appear at the CRYP position. The appearance of a black band at the CONT position is required for the test result to be valid. It indicates that the colloidal dye conjugate is intact and that proper capillary flow has occurred. 

SPECIMEN:

Preferred sample types:

1.  Unpreserved Solid, semi-solid or liquid stool samples 
2.  Stool in Cary-Blair, C&S, Formalin, SAF, or MIF transport media 
Undesirable specimens:

1. Samples in PVA or Ecofix preservatives are not suitable.

2. Colonic washes, aspirates or other diluted sample types.
Interfering substances:
Stool samples contaminated by products with an oily or particulate base (eg. Barium, mineral oil, etc.) could interfere with the test and are not recommended.

Collection:
Stool specimens collected for ova and parasite examination can be used in the Crypto/Giardia assay. The samples should be collected in clean, leak-proof plastic containers. 

Samples collected in SAF, 10% formalin, MIF, Cary-Blair, C&S, or Stuart’s transport media are the preferred media for specimen collection, transport and test. It is recommended that specimens collected in MIF fixative be tested before the addition of Iodine. Samples in PVA are not suitable.
Fresh (unpreserved) samples 

Solid, semi-solid or liquid samples are acceptable but must be diluted 1:4 in an acceptable transport media before running the test. If transport media is not available, distilled or deionized water may be used. In cases of solid or semi-solid samples, mix the specimen using a wooden applicator or equivalent prior to dilution. 

To make a 1:4 dilution: 

• Remove the equivalent of 1 mL mixed specimen and place into 3 mLs of acceptable transport media. 

• Mix thoroughly and proceed with test. 

NOTE:  

· Cary-Blair, C&S, and MIF may interfere with certain confirmatory test methods. Alternative transport media for patient specimens may be recommended for confirmatory testing. As with all test methods, product literature for product performance claims and limitations should be reviewed prior to use. 

· Fresh samples and specimens in Stuart’s media should be tested as soon as possible after collection, as storage for these conditions has not been validated.  

	Formalin and SAF preserved specimens can be stored frozen (-20 C or -70 C), refrigerated (2 – 8 C) or at room temperature (20 – 30 C) and should be tested within two months after collection.
	Cary-Blair or C&S diluted samples can be stored refrigerated (2 – 8 C) and tested within two weeks after collection or frozen (-20 C) and tested within two months.  


	MIF preserved specimens can be stored frozen (-20 C), refrigerated (2 – 8 C) or at room temperature (20 – 30 C) and should be tested within two months after collection.


This facility’s procedure for specimen collection is: _________________________________ __________________________________________________________________________
This facility’s procedure for transporting specimens is: ______________________________

 __________________________________________________________________________
This facility’s procedure for rejected specimens is: _________________________________ __________________________________________________________________________

MATERIALS AND EQUIPMENT:
Materials:

1. Foil pouched Crypto/Giardia test devices consisting of: a) a membrane coated with an avidin derivative, mouse anti-Cryptosporidium and goat anti-mouse IgG, b) pad materials, desiccant and a plastic housing. (30 per test kit)

2. Sample Treatment Buffer: Buffer solution with detergent. WARNING: contains <0.1% azide. (2.8mL)
3. Conjugate Reagent A: Biotinylated rabbit anti-Giardia in diluent buffer with carrier protein and detergent. WARNING: contains <0.1% azide. (1.8mL)
4. Conjugate Reagent B: Colloidal dye labeled monoclonal antibodies to Crypto/Giardia in diluent buffer with carrier protein and detergent. WARNING: contains <0.1% azide. (1.8mL)
5. Specimen transfer pipettes. (1 bag of 30)
6. Specimen dilution tubes. (1 bag of 30)
Equipment:

1. Vortex

2. Interval timer
3. Disposable latex gloves
4. Meridian Bioscience Cryptosporidium/Giardia Controls (Product Code 750820).  

5. Marking pen
6. Specimen collection and transport devices. SAF (900212), 10% formalin (900412), C&S (900612), Cary-Blair, MIF.
7. Test tubes

PERFORMANCE CONSIDERATIONS:

1.
Allow kit components and specimens to equilibrate to room temperature before use. Mix liquid reagents by inverting several times before use. Return kit to the refrigerator promptly after each use. 

2.
Do not unpouch the test devices until ready for use. 

3.
Several tests may be run at the same time. Use separate dilution tubes and pipettes for each specimen. 

4.
To prevent possible contamination, avoid touching the dispensing tip of the Sample Treatment Buffer, Conjugate Reagent A and Conjugate Reagent B dropper bottles to the dilution tubes, pipettes, test devices or anything that has come into direct contact with patient specimens. 

5.
To minimize reagent waste and better control the size of drops dispensed from the reagent bottles, the following steps are suggested: 

• 
Store reagent bottles in upright position. 

• 
After removing the cap, thoroughly wipe tip with a laboratory wipe making sure all liquid is removed

• 
Hold bottles in a near vertical position for dispensing. If running multiple samples, allow air to enter the bottle before dispensing drops for the next test. 

• 
If liquid is observed on the tip at any time, wipe clean with a lab wipe prior to dispensing drops. 

• 
Do not vigorously shake or vortex reagents. 
6.
Do not concentrate patient specimens. When sampling, use the liquid portion of the specimen. It is recommended that particulates be allowed to settle so that the sample can be more easily pipetted. 

7.
All fresh or unpreserved specimens, including liquid samples, must be diluted approximately 1:4 in one of the acceptable transport media prior to use with this assay (Refer to Directional Insert, SPECIMEN COLLECTION and HANDLING section). 

8.
The sequence of reagent additions to the sample tube allows the user to visually monitor the procedure: Sample Treatment Buffer is added to an empty tube; the stool specimen is visually observable; Conjugate Reagent A has a red dye incorporated in it; and Conjugate Reagent B is black in color. 

STORAGE REQUIRMENTS
Store kit refrigerated 2 to 8 C (36 to 46 F) and return kit to the refrigerator promptly after each use. DO NOT FREEZE.
At this facility, kits are stored:_______________________________________________ ​​​​​                                                                                                           
CALIBRATION:

Not applicable to this assay.
QUALITY CONTROL:

Several features are incorporated into the Crypto/Giardia test as routine quality checks: 

1.
The appearance of a control band at the CONT position verifies that a functionally intact colloidal dye conjugate has been added to the device, that the control line antibody is functionally active and that adequate capillary flow has occurred. 

2.
Characterized formalin preserved patient specimens may be used as routine external positive and negative controls. Characterized samples should be run as test specimens according to the procedures described below. 

3.
Quality Control requirements should be established in accordance with local, state and federal regulations or accreditation requirements. Minimally Meridian Bioscience recommends that positive and negative external controls be run with each new lot and with each new untrained operator. 

QC Testing Frequency and Documentation:

For this facility, External QC is run:______________________________________________                                                                                                   

Results of External QC and action(s) taken when control results are unacceptable are documented:________________________________________________________________

TEST PROCEDURE:

This test should be performed by qualified personnel per local regulatory requirements.
Preparation: Bring specimens and reagents to room temperature (20-25 C) before testing 

Specimen Preparation:

1.
Remove the test device from the pouch and place on a flat surface; place a Specimen Dilution Tube into the kit workstation holder or into a suitable test tube rack. 

2.
Add two drops of Sample Treatment Buffer to the Specimen Dilution Tube. 

3.
Use the Specimen Transfer Pipette to aspirate the aqueous patient stool specimen: squeeze the bulb of the pipette, insert the open end into the sample and release the pressure on the bulb while holding the pipette in the sample. Draw sample to the 60 µL calibration line. Transfer the contents of the pipette barrel into the Specimen Dilution Tube. 


NOTE: All fresh or unpreserved specimens, including liquid samples, must be diluted approximately 1:4 in one of the acceptable transport media prior to use with this assay (Refer to Directional Insert, SPECIMEN COLLECTION and HANDLING section). If transport media is not available, distilled or deionized water may be used.

4.
Mix/Invert Conjugate Reagent A. Add two drops of Conjugate Reagent A to the tube. 
5.
Mix/Invert Conjugate Reagent B. Add two drops of Conjugate Reagent B to the tube. 

6.
Mix the sample by manual swirling or by vortexing
This facility’s procedure for specimen preparation is:___________________________  ____________________________________________________________________

This facility’s procedure for sample labeling is:_______________________________ ____________________________________________________________________
Test Procedure
1.   Pour the entire contents of the tube containing the prepped sample into the sample well of the test device. 

2.
Visually read the test results at 10 min. Results are invalid after 15 minutes. 

CALCULATIONS:

There are no calculations associated with this procedure.
INTERPRETATION OF RESULTS

Note: Visible test lines in any shade of grey or black only should be read as positive. 

Test lines in shades of yellow or brown should not be read as positive, but considered an invalid result.

Positive for Cryptosporidium 

The presence of grey-black bands at the CRYP and the CONT positions indicates that Cryptosporidium antigen has been detected. Visible test lines, in shades of grey or black only, should be read as positive. The intensity of the grey-black bands can vary from faint to strong. There is no direct correlation between the intensity of the test line with the severity of the infection. 

[image: image1.emf]
Positive for Giardia 

The presence of grey-black bands at the GIAR and the CONT positions indicates that Giardia antigen has been detected. Visible test lines, in shades of grey or black only, should be read as positive. The intensity of the grey-black bands can vary from faint to strong. There is no direct correlation between the intensity of the test line with the severity of the infection. 

[image: image2.emf]
Negative for Cryptosporidium 

No band is visible at the CRYP position and a visible grey-black band present at the CONT position indicates that Cryptosporidium antigen is absent or is below detectable levels. The GIAR position can have either a visible grey-black band or no grey-black band.

[image: image3.emf]
Negative for Giardia 

No band is visible at the GIAR position and a visible grey-black band present at the CONT position indicates that Giardia antigen is absent or is below detectable levels. The CRYP position can have either a visible grey-black band or no grey-black band.

[image: image4.emf]
Invalid Results 

If no band appears at the CONT position or incomplete or beaded bands appear at the CRYP or GIAR positions, the result is considered invalid. In situations where adequate flow does not occur due to excessive particulate matter in the specimen, the sample can be diluted two-fold in deionized water or in the same transport medium as it was originally collected and re-run. The test should be repeated using another device. Test lines in colors other than grey or black should be considered invalid. Repeat testing using an alternative method or different sample. If an

alternative method is used, verify the preservative and the age of the sample is appropriate (Refer to Directional Insert, SPECIMEN COLLECTION AND HANDLING section).

[image: image5.emf]
In the event this test becomes inoperable, this facility’s course of action for patient samples is:_______________________________________________________________________

REPORTING OF RESULTS 

Negative test for Cryptosporidium:  Report test result as “Cryptosporidium antigen is absent or below detectable levels.”  

Negative test for Giardia:  Report test result as “Giardia antigens is absent or below detectable levels.” 
 Positive test for Cryptosporidium:  Report test result as “Cryptosporidium antigen detected.”
Positive test for Giardia:  Report test result as “Giardia antigen detected.”

This facility’s procedure for patient result reporting is:__________________________________ ____________________________________________________________________________________________________________________________________________________________                                                                         
EXPECTED VALUES:

The prevalence of Cryptosporidium and Giardia is variable among different populations and geographic areas. Giardia incidence in developed countries is approximately 2-5%; Cryptosporidium incidence in Europe and North America is about 1-3%. Higher prevalence rates may be present in children and in the immunosuppressed.

ASSAY REACTIVITY:

A panel of 15 samples consisting of five replicates of a Giardia low level positive sample, five replicates of a Cryptosporidium low level positive sample and five replicates of a negative sample were run on five separate occasions by personnel at two diagnostic parasitology laboratories. Intra-assay reproducibility was demonstrated by 100% agreement (0% CV) among all of the replicate samples within each run. Inter-assay reproducibility was demonstrated by 100% agreement in the results among all of the ten separate occasions that the test panel was run. 

test for interfering substances: 

The Cryptosporidium and Giardia assay was run on stool specimens documented to be positive for other parasites by microscopy.

The Giardia-specific part of the assay showed no cross-reactivity to the following organisms:

	Ascaris lumbricoides (1)
Blastocystis hominis (58)

Chilomastix mesnili (5)

Cryptosporidium parvum (121)

Cyclospora cayetanensis (1)

Dientamoeba fragilis (14)

Endolimax nana (28)
	Entamoeba coli (14)
Entamoeba hartmanni (12)

Entamoeba histolytica/dispar (14)

Enterobius vermicularis (1)

Enteromonas hominis (2)

Hookworm (1)

Hymenolepsis nana (3)
	Iodamoeba bütschlii (13)
Microsporidia (1)

Strongyloides sterocoralis (2)

Taenia sp. (1)

Trichomonas hominis (1)




The numbers in parentheses represent the number of samples tested for each organism.

The Cryptosporidium-specific part of the assay showed no cross-reactivity to the following organisms:

	Ascaris lumbricoides (1)

Blastocystis hominis (92)

Chilomastix mesnili (6)

Cyclospora cayetanensis (1)

Dientamoeba fragilis (16)

Endolimax nana (48)
	Entamoeba coli (21)

Entamoeba hartmanni (20)

Entamoeba histolyticaldispar (19)

Enteromonas hominis (2)

Giardia lamblia (132)

Hookworm (1)
	Hymenolepsis nana (4)

Iodamoeba bütschlii (12)

Microsporidia (1)

Strongyloides sterocoralis (2)

Taenia sp. (2)

Trichomonas hominis (1)


The numbers in parentheses represent the number of samples tested for each organism.

LIMITATIONS OF THE PROCEDURE:
1.
As with all diagnostic procedures, the results obtained with the ImmunoCard STAT! Crypto/Giardia test should be used in conjunction with other clinical information available to the physician. 

2.
Negative results can occur in samples containing levels of antigen below the lower limits of detection of the assay. Multiple specimens collected over several days can be tested for patients suspected of being positive for Giardia or Cryptosporidium. 

3.
The test is designed for use with stool samples collected in an acceptable transport media. The use of colonic washes, aspirates or other diluted sample types has not been established and could affect the performance of the assay. Stool samples contaminated by products with an oily or particulate base (eg. Barium, mineral oil, etc.) could interfere with the test and are not recommended.

PERFORMANCE CHARACTERISTICS:
Refer to Directional Insert – Meridian Bioscience ImmunoCard Stat Crypto/Giardia
REFERENCES:
Refer to Directional Insert – Meridian Bioscience ImmunoCard Stat Crypto/Giardia
Meridian Bioscience

3471 River Hills Drive

Cincinnati, OH 45244 USA



Ph:  (800) 343-3858, (513) 271-3700

0095023 Rev. H 07/15

